Ahmad Barakat | ITEC 422 | Wireshark Lab 4

- What is the IP address of the client?
192.168.1.100

3. What are the source and destination IP addresses and TCP source and destination ports on the IP datagram carrying this HTTP GET?

Source IP: 192.168.1.100

Destination IP: 64.233.169.104

TCP source: 4335

TCP destination: 80

4. At what time is the corresponding 200 OK HTTP message received from the Google server? What are the source and destination IP addresses and TCP source and destination ports on the IP datagram carrying this HTTP 200 OK message?

I don’t see a 200 OK HTTP message.

5. At what time is the client-to-server TCP SYN segment sent that sets up the connection used by the GET sent at time 7.109267? What are the source and destination IP addresses and source and destination ports for the TCP SYN segment? What are the source and destination IP addresses and source and destination ports of the ACK sent in response to the SYN. At what time is this ACK received at the client?

At 7.075657. Dest IP: 64.233.169.104, Source IP: 192.168.1.100, 4335 -> 80

Time: 7.108986	Source: 64.233.169.104	Dest: 192.168.1.100

80 → 4335 [SYN, ACK]

6. At what time does this message appear in the NAT_ISP_side trace file? What are the source and destination IP addresses and TCP source and destination ports on the IP datagram carrying this HTTP GET (as recorded in the NAT_ISP_side trace file)? Which of these fields are the same, and which are different, than in your answer to question 3 above?

Time: 6.871656	Source: 71.192.34.104 Dest: 64.233.169.104 TCP: 4337 → 80 The destination is the same. As well as the TCP destination port.

7. Are any fields in the HTTP GET message changed? Which of the following fields

in the IP datagram carrying the HTTP GET are changed: Version, Header Length,

Flags, Checksum. If any of these fields have changed, give a reason (in one sentence)

stating why this field needed to change.

The length is different, because the length of the message can’t be the same for both unless by

coincidence.

8. In the NAT_ISP_side trace file, at what time is the first 200 OK HTTP message

received from the Google server? What are the source and destination IPaddresses

and TCP source and destination ports on the IP datagram carrying thisHTTP 200 OK

message? Which of these fields are the same, and which are different than your answer

to question 4 above?

I cannot find the HTTP 200 OK message. This was a simple task in the first lab and now I am unable to see it anywhere even by search (http.response.code == 200)

9. In the NAT_ISP_side trace file, at what time were the client-to-server TCP SYN segment and the server-to-client TCP ACK segment corresponding to the segments in question 5 above captured? What are the source and destination IP addresses and source and destination ports for these two segments? Which of these fields are the same, and which are different from your answer to question 5 above?

Time: 6.613690	Source: 71.192.34.104	Dest: 64.233.169.104 TCP 4337 → 80 [SYN]

Time: 6.668189	Source: 64.233.169.104	Dest: 71.192.34.104    TCP 80 → 4338 [SYN, ACK]

The corresponding source and destination are the same. The length is the same, the destination ports are interchanged but that makes sense in this case.